Sample Concentrator—ND200-1

Description:

ND200-1 Sample Concentrator is with microprocessor controlled and PID fuzzy controlled technique. Instead of traditional water bath device, the heat-conducting medium is high purity aluminum. The Test tubes are held in a block, and blowing a large number of samples at one time. It can be used in residue analysis, commodity inspection, food, environment, pharmacy and biological product and other industries.

Features:

◆ High precision and wide range of tamp control.
◆ Blowing a large number of samples at one time.
◆ Blowing each sample independently.
◆ Amount of airflow can be adjusted.
◆ Easy operation; safe and reliable to use.

Applications:

- Analysis of pesticide residues: vegetables, fruits, cereals, plant tissue.

- Environmental analysis: such as drinking water, groundwater and polluted water samples.

- Biological analysis: such as hormone analysis, sample preparation of liquid phase and gas phase and mass spectrometry analysis.

- Food and beverage: such as milk, wine, beer, etc.

- Pharmaceutical drug testing: such as traditional Chinese medicine pharmaceutical, drug screening.

Parameters:

| Model | ND200-1 |
| --- | --- |
| Temp range | RT.+5℃-160℃ |
| Temp accuracy | ±0.5℃(@40℃) |
| Temp accuracy | ±1℃(@120℃) |
| Temp uniformity | ±0.5℃ |
| Display accuracy | 0.1℃ |
| Timing range | 1min-99h59min/∞ |
| Heating time | ≤15 min(from 25℃ to 160℃) |
| Needle length | 150mm |
| Nitrogen flow rate | 15L/min |
| Nitrogen pressure | 0.1Mpa |
| Block quantity | 1 |
| Max power | 300W |
| Voltage | AC220V or AC110V, 50/60Hz |
| Dimension(mm) | 260X220X475 |
| Weight(kg) | 6.0 |

Optional Blocks:

| Type | Description | Type | Description |
| --- | --- | --- | --- |
| MD50 | 12x10mm | MD54 | 12x16mm |
| MD51 | 12x12mm | MD55 | 12x19mm |
| MD52 | 12x13mm | MD56 | 12x20mm |
| MD53 | 12x15mm | --- | --- |