Centrifuge for Microplates—MPC2800

Introduction

MPC2800 Centrifuge for Microplates is designed for quick spins of samples in PCR plates. Primarily designed for use with 96 or 384 well plates and small volumes, it will accommodate skirted, non skirted and semi skirted styles.

Features:

- Easy to use, real-time display of all operating information.

- Countdown mode support.

- DC brushless motor, low noise, small interference, maintenance-free.

- When opening the cap will automatically stop running.

- Fast acceleration from standstill to maximum speed fully in just 10sec.

- Braking time is short, from the highest speed to a complete standstill in just 4 sec.

Parameters

| Model | MPC2800 |
| --- | --- |
| Rotating speed | 2200~2800rpm |
| Timing range | 15sec ~1min59sec |
| Max(RCF) | 550g |
| Acceleration time | ≤10sec(from 0 to 2800rpm) |
| Brake time | ≤4sec(from 2800rpm to 0) |
| Lid Open Auto Stop | Yes |
| Short run | Yes |
| Capacity | 2 PCR plates |
| Power | 45W |
| Voltage | AC100~240V/50-60Hz |
| Fuse | 250V/3A Ф5×20 |
| Dimension(mm) | 260x240x226 |
| Weight(kg) | 3.3 |