Mini Dry Bath Incubator—MiniBox-C

Description:

Minibox-C is a microprocessor-controlled product. It is widely used in preservation and reactions of sample, DNA amplification and initial denaturation of electrophoresis and so forth. It fits almost anywhere and can even be used in cars, boats or wherever a 12 volt power source is available.

Features:

- Energy-efficient design.

- Applicable to 12V power adapter.

- Accurate to ± 0.3℃.

- Process of experiment is visible.

- Electronic calibration. User can reset to any local standard.

- Power supply provided is UL, CSA and CE approved.

- Custom blocks made to fit any sample holder.

Parameters:

| Type Parameters | Minibox-C |
| --- | --- |
| Temperature range | 0℃～100℃ |
| Temp. control range | RT.-15℃～100℃ |
| Timing range | 1sec～5999sec/∞ 1min～5999min/∞ |
| Temp. accuracy | ≤±0.3℃* |
| Display accuracy | 0.1℃ |
| Temp. uniformity | ≤±0.3℃* |
| Heating time | ≤20min(from 25℃ to 100℃)* |
| Cooling time 1 | ≤20min(from 100℃ to 25℃)* |
| Cooling time 2 | ≤25min(from RT. to RT.-15℃)* |
| AUTO-4℃ | Yes* |
| Multi points running | Yes(Max. 5points) |
| Multi cycle | Yes(Max. 99times) |
| Auto heating | Yes |
| Auto running | Yes |
| Auto resume to run | Yes |
| Input power | DC12V |
| Max. power | 60W |
| Dimension(W×D×H) | 145×113×140mm |
| Net weight | 1.0kg |

*Test condition：RT.25℃±2℃, use 15x1.5ml block, use mercurial thermometer, accuracy

is ±0.1℃

Type        Description

MiniBoxA    15×2.0ml

MiniBoxB    15×1.5ml

MiniBoxC    24×0.5ml

MiniBoxD    32×0.2ml (for 0.2ml PCR 8-tube strip)

MiniBoxF    32×0.2ml (for 0.2ml Elisa 8-well strip)