Orbital Shaker-Incubator—TOS20

Description:

Orbital Shaker-Incubator is a compact bench–top Shaker–Incubator used for mixing of biological liquids as well as for incubation and cultivation of biological liquids according to the operator set program. Seven interchangeable platforms allow it mix and culture samples in flask, petri dish, tubes, Elisa plates, and etc. Built–in microprocessor thermo controller provides constant temperature control in the incubator chamber. It is extremely easy to operate, with very straightforward setup of temperature, speed and time.

Features:

- Instantly display operation information(time, temperature and speed) on

the LCD screen.

- Gentle and reliable shaking with direct brushless DC motor.

- Running safely –-heating and shaking will be automatically stopped once

the lid is open.

- Seven interchangeable platforms can be chosen for variety demands.

- Auto restart in case of power failure.

- Over-heating protection device is included.

- Built-in temperature calibration function.

- Compact, only 421mmx320mm(DXW)

Parameters:

| Type Parameter | MS100 |
| --- | --- |
| Temperature setting range | 0°C ~60°C |
| Temperature control range | RT.+5°C ~60°C |
| Time range | 1min ~ 99h59min/∞ |
| Temp. accuracy | ≤±0.5°C(@37°C) |
| Display accuracy | 0.1°C |
| Temp. uniformity | ≤±0.5°C(@37°C) |
| Shaking range | 50~300rpm |
| Orbit | 20mm |
| Platform dimension | 230x230mm |
| Internal chamber height | 182mm |
| Load capacity | 4.0kg |
| Heating time | ≤15min(from 25°C to 60°C) |
| Auto resume to run | Yes |
| Power | 350W |
| Voltage | AC220V/50-60HZ |
| Fuse | 250V 3A Ф5×20 |
| Dimension(mm) | 421x320x338 |
| Weight(kg) | 18.5 |

Optional Platforms

| Type | Description |
| --- | --- |
| PF-1 | Platform with clamps for 5×250ml conical flask |
| PF-2 | Platform with clamps for 9×100ml conical flask |
| PF-3 | Platform with clamps for 10×50ml tube |
| PF-4 | Platform with clamps for 18×15ml tube |
| PF-5 | Platform with spring holder |
| PF-6 | Platform for 4pcs Elisa plates or deep-well plates |
| PF-7 | Flat platform for 5pcs petri dishes |