Dry Bath Incubator——ES2000

Description:

ES2000 Dry Bath Incubator is a microprocessor-controlled dry block heating system with highly precise temperature control, making it an alternative to the traditional water bath devices. Timing flexible, constant temperature function and two-group independent temperature control mode are effectively improving the efficiency of the laboratory work. It is the ideal equipment, applying to kinds of sample incubation, catalyze, save and etc.

Features:

- LCD display, simple interface. Easy to set up and use.

- Accurate temperature control, high stability.

- Unique two-group(A/B) temperature control realizes running independently

or simultaneously.

- One operation interface can realize two-group temperature and timing

setting independently or simultaneously.

- Different kinds of aluminum blocks are available. Block can be customized.

- Automatic preheating function.

- Auto resume to run after power failure.

- Automatic running function.

- Built-in temperature calibration function.

- Built-in over-heating protection device, safe and reliable.

Parameters:

| Model | ES2000 |
| --- | --- |
| Temp setting range | 0°C ~100°C |
| Temp control range | RT+5°C ~100°C |
| Timing range | 1min ~ 99h59min/∞ |
| Temp accuracy | ≤±0.3°C*@37°C |
| Display accuracy | 0.1°C |
| Temp uniformity | ≤±0.3°C@37°C * |
| Heating time | ≤15min(from 25°C~100°C) |
| Auto heating | Yes |
| Auto running | Yes |
| Auto resume to run | Yes |
| Temperature control group | 2 |
| Block quantity | 4 |
| Power | 240W |
| Voltage | AC110V or AC220V/50-60Hz |
| Fuse | 250V 3A Ф5×20 |
| Dimension(mm) | 370x254x217 |
| Weight(kg) | 5.0 |

Optional Blocks:

| Model | Capacity | Model | Capacity |
| --- | --- | --- | --- |
| MiniBoxA | 15x2.0ml centrifuge tube | MD40 | 15xΦ10mm |
| MiniBoxB | 15x1.5ml centrifuge tube | MD41 | 12xΦ12mm |
| MiniBoxC | 24x0.5ml centrifuge tube | MD42 | 8xΦ15mm |
| MiniBoxD | 32x0.2ml centrifuge tube | MD43 | 8xΦ16mm |