Orbital Shaker-Incubator—TOS30

Description:

Orbital Shaker-Incubator, combining shaker and incubator, is a compact bench–top shaking incubator used for mixing of biological liquids as well as for incubation and cultivation of biological liquids. Interchangeable platforms allow it mix and culture samples in flask, petri dish, tubes and etc. Built–in microprocessor thermo controller provides constant temperature control in the incubator chamber. It is extremely easy to operate, with very straightforward setup of temperature, speed and time.

Features:

- Instantly display operation information(time, temperature and speed) on

the LCD screen.

- Gentle and reliable shaking with direct brushless DC motor.

- The incubator features acrylic lid to allow easy visibility of and access to

the samples.

- Constant temperature control ensures even temperature distribution in

the chamber.

- Running safely –-heating and shaking will be automatically stopped once

the lid is open.

- Auto restart in case of power failure.

- Over-heating protection device is included.

- Built-in temperature calibration function.

Parameters:

| Type Parameter | TOS30 |
| --- | --- |
| Temperature setting range | 0°C ~60°C |
| Temperature control range | RT.+5°C ~60°C |
| Time range | 1min ~ 99h59min/∞ |
| Temp. accuracy | ≤±0.5°C(@37°C) |
| Display accuracy | 0.1°C |
| Temp. uniformity | ≤±0.5°C(@37°C) |
| Shaking range | 50~250rpm |
| Heating time | ≤20min(from 25°C to 60°C) |
| Orbit | 20mm |
| Platform dimension | 350x350mm |
| Internal chamber height | 325mm |
| Load capacity | 10.0kg |
| Auto resume to run | Yes |
| Power | 600W |
| Voltage | AC220V or AC110V/50-60HZ |
| Fuse | 250V 3A Ф5×20 |
| Dimension(mm) | 632x502x512 |
| Weight(kg) | 41 |

Optional Platforms

| Type | Description |
| --- | --- |
| TOS30-T1 | Platform with clamps for 9×250ml conical flask |
| TOS30-T2 | Platform with clamps for 25×100ml conical flask |
| TOS30-T3 | Flat platform for 16pcs petri dishes |
| TOS30-T4 | Platform with spring holder |