High-speed Mini-centrifuge—MC15K

Introduction

The High-speed Mini-centrifuge is a compact centrifuge capable of achieving centrifugal speeds up to 15000rpm, that is approximately 15080g. It is used for extracting RNA/DNA samples, sedimentation of biological components, biochemical and chemical analysis of micro samples. Automatic imbalance switch-off and locking of lid provide safe operation. The rotor accepts 1.5 to 2.0ml tubes directly, as well as 0.5ml and 0.2ml tubes with optional adapters.

Features:

- LCD provides a clear indication of time and rpm or RCF.

- RPM/RCF setting as required.

- The rotor is made of nylon material, with corrosion resistance, minimizes the temperature increases during longer run at high speed.

- Accelerates and decelerates in just 14seconds.

- Safety operation: rotor imbalance diagnostic; automatic stop.

- Motor overheating protection function.

- A separate momentary short button permits instant spin downs.

- Low sample heating (only 5°C after 30min, at max. speed).

- A brushless motor provides noiseless performance and long service life.

Parameters

| Model | MC15K |
| --- | --- |
| Capacity | 12x1.5/2.0ml centrifuge tubes |
| Speed range | 500~15000rpm |
| RCF range | 16~15080g |
| Timing range | 15sec~99min59sec/∞ |
| Acceleration | 14sec(0~15000rpm) |
| Deceleration | 14sec(15000~1000rpm) |
| Rotor imbalance diagnostic | Yes |
| Motor overheating protection function | Yes |
| Power | 120W |
| Voltage | DC24V/5A |
| Dimension(mm) | 255x194x140mm |
| Weight | 2.3kg |