Mini Thermal Cycler—MTC3200

## 1. Introduction

Mini Thermal Cycler is a compact, accurate and economical thermal cycler, designed to fit lab’s basic thermal cycling needs. The operation of the thermal cycler is controlled by a 5-inch color touch screen. It has a capacity for 32x0.2ml tubes. The wells are arranged in a 4x8 format, allowing the block to accept strips of 8 as well as individual tubes. Excellent block uniformity ensures consistency sample to sample. Fast incubation function meets the experimental needs of denaturation, enzyme digestion / enzyme linkage, ELISA, etc. With a small footprint, the Mini Thermal Cycler will fit almost anywhere.

Features:

- Capacity: 32x0.2ml tubes and 4x8 tube strips

- Heating/cooling method: Peltier

- 5-inch intuitive color touch screen—for easy programming of users.

- 100 typical programs can be stored in memory and easily recalled.

- The block heats and cools very quickly—at rates as fast as 4°C/sec.

- The heated lid is self adjusting—the spring loaded platen in the lid comes firmly into contact with the tops of the tubes to prevent sample evaporation and condensation.

- Temperature of heated lid can be changed or the heating function of the lid can be turned off.

- After a run, samples can be held at 4°C.

- An on-screen report is available at the completion of a run.

## 2. Parameters

| Model | MTC3200 |
| --- | --- |
| Capacity | 32x0.2ml tubes, 4x8 tube strips |
| Temp. setting range | 0℃~100℃ |
| Temp. control range | 4℃~100℃ |
| Temp. accuracy/uniformity | ≤±0.25℃/≤±0.25℃ |
| Max. heating rate | 4℃/sec |
| Max. cooling rate | 2℃/sec |
| Heated lid temperature | RT.+5℃~110℃, user adjustable |
| Temp. display resolution | 0.1℃ |
| Temp. control | Simulated sample or block |
| Display | 5-inch color touch screen, 800x480 Pixel |
| Communication interface | USB(1 port) |
| Program memory | 100 programs |
| Time increments/decrements | 1~120s |
| Temp. increments/decrements | 0.1~9.9℃ |
| Storage at 4℃ | Yes |
| On-screen report | Yes |
| Power | 200W |
| Voltage | AC100~240V, 50-60Hz |
| Fuse | 250V 3A Ф5×20 |
| Dimension(mm) | 264x200x125 |
| Weight(kg) | 2.3 |