Vortex Mixer—MIX2000

Description:

MIX2000 Vortex Mixer is a high efficiency benchtop mixer. It is used for mixing samples like tissues, cells, bacteria liquid and chemical reagents in life science, physical and chemical analysis fields. It is designed to meet most of the requirements of many lab users.

Features:

- Touch operation or continuous mode.

- Variable speed control from 0 to 2800rpm.

- Used for various mixing application with optional adaptors.

- Accept a variety of test tubes, including the tubes up to 50ml.

## Parameters:

| Model | MIX2000 |
| --- | --- |
| Time range | 0sec ~99min59sec/∞ |
| Speed range | 0~2800rpm |
| Speed control mode | Stepless speed regulation |
| Run mode | Touch operation/Continuous |
| Mixing motion | Orbital |
| Orbit | 3mm |
| Motor type | Shaded pole motor |
| Power supply | 60W |
| Voltage | AC220V/50-60Hz |
| Dimension(mm) | 197x135x156mm |
| Weight | 3.4kg |

## Adaptors

| Name | Description | Remark |
| --- | --- | --- |
| Single-tube head | For one tube | Standard |
| Foam block platform | Used with foam block | Standard |
| Foam block A | 4x29mm | Standard |
| Foam block B | 8x15mm | Standard |
| Foam block C | 24x10mm | Standard |
| Foam block D | 37x8mm | Standard |

Note: Foam block platform and foam blocks are only suitable for multiple samples in touch mode mixing.